요구사항 충족 증명 보고서

Automotive_Cluster 프로젝트 요구사항 충족 증명 보고서

요구사항-구현-검증-증거 추적성 기반 공식 기술 검증 보고서

| 항목 | 값 |
| --- | --- |
| 프로젝트 | Automotive_Cluster |
| 워크스페이스 | C:\Users\JunsangS\workspaceS32DS.Power.2.1\Automotive_Cluster |
| 원 요구사항 문서 | MPC5748G_Embedded_Training_Requirements_EN.docx |
| 작성일 | 2026-06-04 |
| 검증 기준 | Inspection, static artifacts, host unit tests, and available logs |

# 문서 목적

본 문서는 Automotive_Cluster 워크스페이스가 원 요구사항 문서의 요구사항을 어느 수준까지 충족하는지, 요구사항 ID별로 구현 위치, 검증 방법, 증거 파일/로그, 판정을 연결하여 제시한다. 증거가 부족한 항목은 충족으로 판정하지 않고 확인 필요로 분류하였다.

# 프로젝트 개요

원 요구사항 문서는 MPC5748G 기반 6개 ECU 교육 프로젝트를 정의한다. 현재 워크스페이스는 ECU6 Gateway + OTA + Security 노드를 중심으로 구성되어 있으며, ECU1~ECU5는 CANoe 시뮬레이터/DBC 지원 자산은 있으나 독립 MPC5748G ECU 구현 증거는 확인되지 않았다.

# 검증 범위

전체 요구사항 96개를 추적성 매트릭스에 포함하였다.

ECU6 활성 범위는 Common 15개, ECU6 16개, ECU6 관련 Integration 4개로 총 35개이다.

ECU1~ECU5 개별 보드 요구사항과 일부 통합 시나리오는 현재 워크스페이스 증거만으로 최종 충족 판정하지 않았다.

# 사용한 자료 및 증거 소스

| 분류 | 경로 | 검토 내용 |
| --- | --- | --- |
| 요구사항 | MPC5748G_Embedded_Training_Requirements_EN.docx | 96개 요구사항 추출 |
| 상태 문서 | Documentation/ecu6_gateway/requirements_status.md | ECU6 활성 범위 35개 판정 기준 |
| 설계/시험 문서 | Documentation/ecu6_gateway/design.md; test_report.md | CAN, GatewayStatus, OTA/security 설계 및 테스트 자산 |
| 소스 코드 | Sources/**/*.c; Generated_Code/**/*.c | 초기화, CAN, gateway, scheduler, logging 구현 |
| CANoe/CMM | CANOE/*; cmm/* | DBC, CAPL, panel, Trace32 스크립트 및 binary report artifact |
| 빌드/테스트 | Debug_FLASH/Automotive_Cluster.elf; host_unit_tests_2026-06-04.txt | 빌드 산출물 및 9/9 host unit test 통과 |

# 요구사항 충족 요약

| 판정 | 수량 | 의미 |
| --- | --- | --- |
| 충족 | 31 | 코드, 문서, 테스트 또는 실행 가능한 증거가 요구사항을 닫기에 충분함 |
| 부분 충족 | 4 | 구현 또는 테스트 자산은 있으나 런타임/외부 도구 증거가 부족함 |
| 미충족 | 0 | 의미 있는 구현 증거가 없음 |
| 확인 필요 | 61 | 현재 워크스페이스만으로는 판정할 증거가 부족함 |

# 요구사항 추적성 매트릭스

| 요구사항 ID | 요구사항 내용 | 충족 여부 | 구현 위치 | 검증 방법 | 증거 파일/로그 | 비고 |
| --- | --- | --- | --- | --- | --- | --- |
| REQ-COM-001 | 각 ECU는 시스템 클록, GPIO, UART, CAN 컨트롤러, 타이머/스케줄러 및 필요한 인터럽트를 초기화해야 한다. | 부분 충족 | Sources/main.c; Sources/services/system_init.c; Sources/drivers/gpio.c; Sources/drivers/can.c | Source inspection; Debug_FLASH artifact inspection; pending S32DS runtime/flash evidence | Debug_FLASH/Automotive_Cluster.elf; Debug_FLASH/Automotive_Cluster.map; cmm/interrupt_evidence.cmm | Runtime stability, UART boot log, heartbeat LED, and Trace32 screenshot evidence are still required. |
| REQ-COM-002 | 각 ECU는 [time][ECUx][module][level] message 형식의 일관된 UART 로그를 제공해야 한다. | 충족 | Sources/services/logger.c; Sources/automotive.h; Sources/services/com_gateway.c | Source inspection of LOG_Print() format and gateway/OTA logging call sites | Sources/services/logger.c; Sources/services/com_gateway.c | Saved UART terminal evidence was not found, but code implements the required format. |
| REQ-COM-003 | 각 ECU는 최소 하나의 주기 CAN 메시지를 송신하고 다른 ECU 또는 CANoe의 CAN 메시지를 수신해야 한다. | 부분 충족 | Sources/drivers/can.c; Sources/services/com_gateway.c; CANOE/*.dbc; CANOE/*.can | Source/DBC/CAPL inspection; readable CANoe verdict export still pending | CANOE/6_ECUs_dbc_0206.dbc; CANOE/ECU6_Gateway_Test.can; CANOE/*.vtestreport | Binary CANoe reports exist, but text-readable PASS/FAIL verdicts were not available. |
| REQ-COM-004 | 각 ECU는 10 ms, 100 ms, 1000 ms 작업 또는 동등한 스케줄러를 구현해야 한다. | 부분 충족 | Sources/services/ms_scheduler.c; Sources/main.c; cmm/scheduler_evidence.cmm | Source inspection; Trace32 scheduler counter capture pending | Sources/services/ms_scheduler.c; cmm/scheduler_evidence.cmm; Debug_FLASH/Automotive_Cluster.map | Scheduler counters are present in source; a rebuild and Trace32 evidence are required for final runtime closure. |
| REQ-COM-005 | 각 ECU는 중요 수신 CAN 메시지 또는 외부 센서 입력의 timeout 감시를 구현해야 한다. | 충족 | Sources/services/com_gateway.c: COM_GatewayCheckTimeouts(), COM_GatewayHandleRxMessage() | Source inspection and host unit test coverage | Tests/host/test_ecu6_gateway_logic.py; Documentation/ecu6_gateway/evidence/host_unit_tests_2026-06-04.txt | Host tests passed; hardware/CANoe runtime evidence remains recommended. |
| REQ-COM-006 | 각 ECU의 primary status 메시지는 4-bit alive counter를 포함해야 한다. | 충족 | Sources/services/com_gateway.c: COM_GatewayBuildStatusMessage(); CANOE/6_ECUs_dbc_0206.dbc | Source and DBC inspection | Sources/services/com_gateway.c; CANOE/6_ECUs_dbc_0206.dbc | GatewayStatus byte 6 carries the 4-bit alive counter. |
| REQ-COM-007 | 각 ECU의 primary status 메시지는 단순 checksum을 포함해야 한다. | 충족 | Sources/services/com_gateway.c: COM_CalculateChecksum(), COM_VerifyChecksum() | Source inspection and host unit test coverage | Tests/host/test_ecu6_gateway_logic.py; host_unit_tests_2026-06-04.txt | Host tests passed for checksum calculation and validation. |
| REQ-COM-008 | 각 ECU는 driver, service, application 계층을 분리해야 한다. | 충족 | Sources/main.c; Sources/drivers/*.c; Sources/services/*.c; Generated_Code/*.c | Source tree review | Sources/drivers; Sources/services; Generated_Code | Driver/service/generated layers are separated. |
| REQ-COM-009 | 여러 동작 모드가 있는 ECU는 상태 기계로 동작을 구현해야 한다. | 충족 | Sources/services/com_gateway.c: gateway, OTA, and security state handling | Source inspection and host unit test coverage | Documentation/ecu6_gateway/design.md; Tests/host/test_ecu6_gateway_logic.py | Gateway, OTA, and security modes are explicitly represented. |
| REQ-COM-010 | 각 멘티는 Trace32용 init_debug.cmm 및 test_debug.cmm을 작성해야 한다. | 충족 | cmm/init_debug.cmm; cmm/test_debug.cmm; cmm/*.cmm | File inspection | cmm/init_debug.cmm; cmm/test_debug.cmm; cmm/interrupt_evidence.cmm | Trace32 screenshots are still required for runtime evidence. |
| REQ-COM-011 | 각 멘티는 담당 ECU에 대해 CAPL 테스트 케이스를 최소 3개 작성해야 한다. | 충족 | CANOE/ECU6_Gateway_Test.can; CANOE/ECU6_OTA_State_Test.can | CAPL file inspection; binary report artifacts observed | CANOE/ECU6_Gateway_Test.can; CANOE/ECU6_OTA_State_Test.can; CANOE/*.vtestreport | Readable CANoe PASS/FAIL export is pending. |
| REQ-COM-012 | 각 멘티는 설계 문서와 테스트 문서를 제공해야 한다. | 충족 | Documentation/ecu6_gateway/design.md; Documentation/ecu6_gateway/test_report.md | Document inspection | Documentation/ecu6_gateway/design.md; Documentation/ecu6_gateway/test_report.md | Design and test documentation exists. |
| REQ-COM-013 | 각 ECU는 입력 또는 메시지가 정상 상태로 복귀할 때 fault recovery를 지원해야 한다. | 충족 | Sources/services/com_gateway.c: timeout/alive/checksum recovery paths | Source inspection and host unit test coverage | Tests/host/test_ecu6_gateway_logic.py; Documentation/ecu6_gateway/design.md | Host tests include alive and timeout recovery. |
| REQ-COM-014 | 각 ECU는 중요 로직을 요구사항 ID와 연결할 수 있는 traceable name/comment를 사용해야 한다. | 충족 | Sources/services/com_gateway.c comments; cmm/gateway_debug.cmm; Documentation/ecu6_gateway/*.md | Traceability review | Sources/services/com_gateway.c; cmm/gateway_debug.cmm; Documentation/ecu6_gateway/requirements_status.md | Requirement IDs appear in code, scripts, and status documents. |
| REQ-COM-015 | 각 ECU는 checksum, filter, warning rule, state transition 등 순수 로직에 대한 host-side unit test를 포함할 수 있다. | 충족 | Tests/host/test_ecu6_gateway_logic.py | Executed Python unittest discovery | Documentation/ecu6_gateway/evidence/host_unit_tests_2026-06-04.txt | 9/9 host-side unit tests passed. |
| REQ-ECU1-001 | ECU1 범위 요구사항 해석: ECU1 반드시 읽기 또는 수신 four door states and 검증 each value. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-002 | ECU1 범위 요구사항 해석: ECU1 반드시 읽기 또는 수신 seatbelt status and include it in BodyStatus. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-003 | ECU1 범위 요구사항 해석: ECU1 반드시 지원 turn signal modes: off, left, right, and hazard. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-004 | ECU1 범위 요구사항 해석: ECU1 반드시 송신 BodyStatus on CAN ID 0x101 주기로 100 ms. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-005 | ECU1 범위 요구사항 해석: ECU1 반드시 set BodyWarning if vehicle speed is greater than 5 km/h and any door is open. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-006 | ECU1 범위 요구사항 해석: ECU1 반드시 set BodyWarning if speed is greater than 10 km/h and seatbelt is not fastened. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-007 | ECU1 범위 요구사항 해석: ECU1 권장 수준으로 debounce physical/simulated switch inputs. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-008 | ECU1 범위 요구사항 해석: ECU1 반드시 검출 input timeout if the selected input source is not updated for more than 500 ms. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-009 | ECU1 범위 요구사항 해석: ECU1 권장 수준으로 recover from input timeout when valid input resumes. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU1-010 | ECU1 범위 요구사항 해석: ECU1 권장 수준으로 제공 a Body Control CANoe panel. | 확인 필요 | No ECU1 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU1.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-001 | ECU2 범위 요구사항 해석: ECU2 반드시 수신 또는 읽기 EngineRPM, ThrottlePercent, and EngineTemp. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-002 | ECU2 범위 요구사항 해석: ECU2 반드시 검증 sensor ranges before using values. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-003 | ECU2 범위 요구사항 해석: ECU2 반드시 calculate VehicleSpeed from RPM or accepted input using a documented formula. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-004 | ECU2 범위 요구사항 해석: ECU2 권장 수준으로 smooth speed changes instead of jumping instantly to target speed. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-005 | ECU2 범위 요구사항 해석: ECU2 반드시 송신 PowertrainStatus on CAN ID 0x201 주기로 50 ms. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-006 | ECU2 범위 요구사항 해석: ECU2 반드시 set EngineWarning when EngineTemp > 105 degC. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-007 | ECU2 범위 요구사항 해석: ECU2 반드시 set EngineCriticalFault when EngineTemp > 120 degC. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-008 | ECU2 범위 요구사항 해석: ECU2 반드시 enter limp mode when EngineCriticalFault is active. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-009 | ECU2 범위 요구사항 해석: ECU2 반드시 검출 sensor timeout after 500 ms without valid input update. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU2-010 | ECU2 범위 요구사항 해석: ECU2 권장 수준으로 제공 a recovery rule for overheat and sensor timeout. | 확인 필요 | No ECU2 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU2.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-001 | ECU3 범위 요구사항 해석: ECU3 반드시 수신 또는 읽기 ambient temperature, light level, battery voltage, and steering angle. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-002 | ECU3 범위 요구사항 해석: ECU3 반드시 검증 range for all sensor inputs. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-003 | ECU3 범위 요구사항 해석: ECU3 반드시 apply a moving average filter to at least one sensor. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-004 | ECU3 범위 요구사항 해석: ECU3 반드시 송신 SensorStatus on CAN ID 0x301 주기로 100 ms. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-005 | ECU3 범위 요구사항 해석: ECU3 반드시 set NightModeRequest when LightLevel < 20 %. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-006 | ECU3 범위 요구사항 해석: ECU3 반드시 set BatteryLow when BatteryVoltage < 11 V. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-007 | ECU3 범위 요구사항 해석: ECU3 반드시 set BatteryOverVoltage when BatteryVoltage > 15 V. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-008 | ECU3 범위 요구사항 해석: ECU3 반드시 검출 sensor timeout when no update is received for more than 500 ms. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-009 | ECU3 범위 요구사항 해석: ECU3 권장 수준으로 검출 a stuck sensor when a value remains unchanged for a configured number of samples while source is active. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU3-010 | ECU3 범위 요구사항 해석: ECU3 권장 수준으로 document I2C/SPI frame format used with Arduino. | 확인 필요 | No ECU3 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU3.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-001 | ECU4 범위 요구사항 해석: ECU4 반드시 receive BodyStatus, PowertrainStatus, SensorStatus, DiagnosticStatus, and GatewayStatus. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-002 | ECU4 범위 요구사항 해석: ECU4 반드시 구현 a cluster state machine with INIT, NORMAL, WARNING, DIAG_MODE, OTA_MODE, and FAIL_SAFE states. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-003 | ECU4 범위 요구사항 해석: ECU4 반드시 송신 ClusterStatus on CAN ID 0x401 주기로 100 ms. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-004 | ECU4 범위 요구사항 해석: ECU4 반드시 display VehicleSpeed and EngineRPM from ECU2. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-005 | ECU4 범위 요구사항 해석: ECU4 반드시 turn on DoorWarningLamp when body warning indicates door open while driving. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-006 | ECU4 범위 요구사항 해석: ECU4 반드시 turn on EngineWarningLamp for ECU2 EngineWarning or CriticalFault. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-007 | ECU4 범위 요구사항 해석: ECU4 반드시 turn on BatteryWarningLamp for low battery or over-voltage from ECU3. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-008 | ECU4 범위 요구사항 해석: ECU4 반드시 enter FAIL_SAFE if PowertrainStatus timeout exceeds 200 ms. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-009 | ECU4 범위 요구사항 해석: ECU4 반드시 enter DIAG_MODE when DiagnosticStatus indicates active diagnostic operation. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-010 | ECU4 범위 요구사항 해석: ECU4 반드시 enter OTA_MODE when GatewayStatus indicates active OTA operation. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-011 | ECU4 범위 요구사항 해석: ECU4 권장 수준으로 prioritize displays when multiple warnings are active. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU4-012 | ECU4 범위 요구사항 해석: ECU4 권장 수준으로 구현 readable UART display updated 주기로 500 ms or 1 s. | 확인 필요 | No ECU4 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU4.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-001 | ECU5 범위 요구사항 해석: ECU5 반드시 receive UDS physical requests on CAN ID 0x7E0 and respond on 0x7E8. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-002 | ECU5 범위 요구사항 해석: ECU5 반드시 지원 Diagnostic Session Control service 0x10 for Default Session 0x01 and Extended Session 0x03. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-003 | ECU5 범위 요구사항 해석: ECU5 반드시 지원 ECU Reset service 0x11 with hard reset sub-function 0x01 or a documented reset simulation. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-004 | ECU5 범위 요구사항 해석: ECU5 반드시 지원 Read Data By Identifier service 0x22. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-005 | ECU5 범위 요구사항 해석: ECU5 반드시 제공 DID 0xF187 Software Version and DID 0xF188 Hardware Version. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-006 | ECU5 범위 요구사항 해석: ECU5 권장 수준으로 제공 DID 0xF190 VIN. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-007 | ECU5 범위 요구사항 해석: ECU5 반드시 지원 Write Data By Identifier service 0x2E for at least one demo configuration DID. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-008 | ECU5 범위 요구사항 해석: ECU5 반드시 지원 Read DTC Information service 0x19 0x02. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-009 | ECU5 범위 요구사항 해석: ECU5 반드시 지원 Clear Diagnostic Information service 0x14 for all DTCs. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-010 | ECU5 범위 요구사항 해석: ECU5 반드시 구현 negative responses for un지원ed service, un지원ed sub-function, request out of range, and conditions not correct. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-011 | ECU5 범위 요구사항 해석: ECU5 권장 수준으로 구현 Security Access service 0x27 using a simple seed/key algorithm. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-012 | ECU5 범위 요구사항 해석: ECU5 반드시 송신 DiagnosticStatus on CAN ID 0x501 주기로 100 ms. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-013 | ECU5 범위 요구사항 해석: ECU5 권장 수준으로 store DTCs based on fault status from ECU2, ECU3, or ECU6. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU5-014 | ECU5 범위 요구사항 해석: ECU5 권장 수준으로 include uds_debug.cmm for Trace32. | 확인 필요 | No ECU5 MPC5748G board source implementation found in this workspace; supporting CAPL simulator: CANOE\ECU5.can | Workspace inspection only; board implementation and execution evidence required | MPC5748G_Embedded_Training_Requirements_EN.docx; CANOE support files where present | 증거 부족 / 확인 필요. Insufficient evidence for the assigned ECU board implementation. |
| REQ-ECU6-001 | ECU6 범위 요구사항 해석: ECU6 반드시 monitor or route allowed CAN IDs 0x101, 0x201, 0x301, 0x401, and 0x501. | 충족 | Sources/drivers/can.c; Sources/services/com_gateway.c | Source/DBC/CAPL inspection | CANOE/6_ECUs_dbc_0206.dbc; CANOE/ECU6_Gateway_Test.can | Allowed IDs 0x101, 0x201, 0x301, 0x401, and 0x501 are monitored. |
| REQ-ECU6-002 | ECU6 범위 요구사항 해석: ECU6 반드시 block or count unknown CAN IDs. | 충족 | Sources/drivers/can.c; Sources/services/com_gateway.c: COM_GatewayHandleRxMessage() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Unknown ID increments blocked counter. |
| REQ-ECU6-003 | ECU6 범위 요구사항 해석: ECU6 반드시 검증 checksum for monitored primary messages. | 충족 | Sources/services/com_gateway.c: COM_VerifyChecksum() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Checksum mismatch sets error/counting behavior. |
| REQ-ECU6-004 | ECU6 범위 요구사항 해석: ECU6 반드시 monitor alive counters for selected ECUs. | 충족 | Sources/services/com_gateway.c: COM_UpdateAliveMonitor() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Alive freeze detection and recovery tested on host. |
| REQ-ECU6-005 | ECU6 범위 요구사항 해석: ECU6 반드시 구현 OTA states: IDLE, START, RECEIVING, VERIFYING, READY_TO_ACTIVATE, SUCCESS, FAILED. | 충족 | Sources/services/com_gateway.c: OTA_ApplyCommandState(); CANOE/6_ECUs_dbc_0206.dbc | Source/DBC/CAPL inspection | CANOE/ECU6_OTA_State_Test.can | OTA states are implemented and exposed in GatewayStatus. |
| REQ-ECU6-006 | ECU6 범위 요구사항 해석: ECU6 반드시 지원 OTA_REQUEST CAN ID 0x650 and OTA_RESPONSE CAN ID 0x651. | 충족 | Sources/drivers/can.c; Sources/services/com_gateway.c: OTA_HandleRequestMessage() | Source/DBC/CAPL inspection | CANOE/6_ECUs_dbc_0206.dbc; CANOE/ECU6_OTA_State_Test.can | 0x650 request and 0x651 response are implemented. |
| REQ-ECU6-007 | ECU6 범위 요구사항 해석: ECU6 반드시 지원 OTA commands START, DATA, END, ACTIVATE, and ABORT. | 충족 | Sources/services/com_gateway.c: OTA_ApplyCommandState() | Source/CAPL inspection | CANOE/ECU6_OTA_State_Test.can | START/DATA/END/ACTIVATE/ABORT paths and invalid sequence NACK are implemented. |
| REQ-ECU6-008 | ECU6 범위 요구사항 해석: ECU6 반드시 store OTA data in a bounded buffer of at least 256 bytes. | 충족 | Sources/services/com_gateway.c: OTA_StoreDataPayload() | Source/CAPL inspection | CANOE/ECU6_OTA_State_Test.can | Bounded 256-byte buffer and overflow rejection are implemented. |
| REQ-ECU6-009 | ECU6 범위 요구사항 해석: ECU6 반드시 verify OTA data using CRC/checksum before activation. | 충족 | Sources/services/com_gateway.c: OTA_VerifyDataChecksum() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Payload XOR checksum success/failure is tested. |
| REQ-ECU6-010 | ECU6 범위 요구사항 해석: ECU6 반드시 require security unlock before accepting OTA_START. | 충족 | Sources/services/com_gateway.c: OTA_ApplyCommandState() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | START is rejected before security unlock. |
| REQ-ECU6-011 | ECU6 범위 요구사항 해석: ECU6 반드시 구현 seed/key security access for OTA. | 충족 | Sources/services/com_gateway.c: Security_GenerateSeed(), Security_CalculateKey() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Seed/key unlock and wrong-key handling are tested. |
| REQ-ECU6-012 | ECU6 범위 요구사항 해석: ECU6 권장 수준으로 구현 OTA_DATA authentication byte. | 충족 | Sources/services/com_gateway.c: OTA_VerifyDataAuth() | Source/CAPL inspection | CANOE/ECU6_OTA_State_Test.can; cmm/ota_debug.cmm | OTA_DATA AuthByte rejection is implemented. |
| REQ-ECU6-013 | ECU6 범위 요구사항 해석: ECU6 권장 수준으로 lock OTA after three failed security attempts. | 충족 | Sources/services/com_gateway.c: Security_RecordFailure(), Security_IsLockoutActive() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Three failed attempts trigger lockout and recovery timeout. |
| REQ-ECU6-014 | ECU6 범위 요구사항 해석: ECU6 반드시 송신 GatewayStatus on CAN ID 0x601 주기로 100 ms. | 충족 | Sources/services/com_gateway.c: COM_GatewayBuildStatusMessage(); Sources/drivers/can.c: vCANTask() | Source/DBC inspection | Debug_FLASH/Automotive_Cluster.map; CANOE/6_ECUs_dbc_0206.dbc | GatewayStatus 0x601 is built and scheduled every 100 ms. |
| REQ-ECU6-015 | ECU6 범위 요구사항 해석: ECU6 권장 수준으로 update ActiveSWVersion after successful OTA activation. | 충족 | Sources/services/com_gateway.c: OTA_AdvanceActiveSwVersion() | Source inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Version increments only after verified activation. |
| REQ-ECU6-016 | ECU6 범위 요구사항 해석: ECU6 권장 수준으로 제공 gateway_debug.cmm, ota_debug.cmm, and security_debug.cmm. | 충족 | cmm/gateway_debug.cmm; cmm/ota_debug.cmm; cmm/security_debug.cmm | File inspection | cmm/*.cmm | Trace32 scripts watch gateway, OTA, and security variables. |
| REQ-INT-001 | 통합 요구사항 해석: All six boards 반드시 run together on the same CAN network without CAN ID conflict. | 부분 충족 | CANOE/ECU1.can..ECU5.can; Sources/drivers/can.c; Sources/services/com_gateway.c; Configuration1.cfg | Static integration setup inspection; full CANoe run pending | Configuration1.cfg; CANOE/6_ECUs_dbc_0206.dbc; CANOE/*.can | Full six-board CANoe/bench trace is still required. |
| REQ-INT-002 | 통합 요구사항 해석: The system 반드시 지원 a Normal Driving 시나리오. | 확인 필요 | No complete full-system implementation evidence found in this workspace | Workspace inspection only; full CANoe/bench execution evidence required | Configuration1.cfg; CANOE/*.dbc; CANOE/*.can | 증거 부족 / 확인 필요. Full integration execution or exported CANoe report is required. |
| REQ-INT-003 | 통합 요구사항 해석: The system 반드시 지원 Door Open While Driving 시나리오. | 확인 필요 | No complete full-system implementation evidence found in this workspace | Workspace inspection only; full CANoe/bench execution evidence required | Configuration1.cfg; CANOE/*.dbc; CANOE/*.can | 증거 부족 / 확인 필요. Full integration execution or exported CANoe report is required. |
| REQ-INT-004 | 통합 요구사항 해석: The system 반드시 지원 Engine Overheat 시나리오. | 확인 필요 | No complete full-system implementation evidence found in this workspace | Workspace inspection only; full CANoe/bench execution evidence required | Configuration1.cfg; CANOE/*.dbc; CANOE/*.can | 증거 부족 / 확인 필요. Full integration execution or exported CANoe report is required. |
| REQ-INT-005 | 통합 요구사항 해석: The system 반드시 지원 Sensor Fault 시나리오. | 확인 필요 | No complete full-system implementation evidence found in this workspace | Workspace inspection only; full CANoe/bench execution evidence required | Configuration1.cfg; CANOE/*.dbc; CANOE/*.can | 증거 부족 / 확인 필요. Full integration execution or exported CANoe report is required. |
| REQ-INT-006 | 통합 요구사항 해석: The system 반드시 지원 Diagnostic 시나리오. | 확인 필요 | No complete full-system implementation evidence found in this workspace | Workspace inspection only; full CANoe/bench execution evidence required | Configuration1.cfg; CANOE/*.dbc; CANOE/*.can | 증거 부족 / 확인 필요. Full integration execution or exported CANoe report is required. |
| REQ-INT-007 | 통합 요구사항 해석: The system 반드시 지원 OTA Success 시나리오. | 충족 | Sources/services/com_gateway.c; CANOE/ECU6_OTA_State_Test.can | Source/CAPL inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | OTA success logic, activation, and version update are covered by host tests. |
| REQ-INT-008 | 통합 요구사항 해석: The system 반드시 지원 OTA Fail 시나리오. | 충족 | Sources/services/com_gateway.c; CANOE/ECU6_OTA_State_Test.can | Source/CAPL inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Wrong checksum failure keeps version unchanged. |
| REQ-INT-009 | 통합 요구사항 해석: The system 반드시 지원 Gateway Security 시나리오. | 충족 | Sources/services/com_gateway.c; CANOE/ECU6_Gateway_Test.can | Source/CAPL inspection and host unit test | Tests/host/test_ecu6_gateway_logic.py | Invalid ID and checksum error behavior is implemented and host-tested. |

# 기능별 상세 증빙

그림 1. 시스템 아키텍처: ECU6, CANoe, Trace32, S32DS 산출물의 증거 연결

그림 2. 정적 구조: Generated/SDK, Driver, Service, Test/Evidence 계층

그림 3. 동적 흐름: CAN 수신, gateway 검증, OTA/security, 0x601 송신

| 기능 영역 | 구현 위치 | 주요 증거 |
| --- | --- | --- |
| 초기화/RTOS | Sources/main.c; Sources/services/system_init.c; Generated_Code/FreeRTOSConfig.h | clock/pin/UART/CAN 초기화, FreeRTOS task 생성, 32 KB heap 설정 |
| CAN gateway | Sources/drivers/can.c; Sources/services/com_gateway.c | 0x101~0x501 수신, 0x601 송신, unknown ID catch-all, timeout/checksum/alive 감시 |
| OTA/security | Sources/services/com_gateway.c; CANOE/ECU6_OTA_State_Test.can | 0x650/0x651, START/DATA/END/ACTIVATE/ABORT, seed/key, AuthByte, lockout, ActiveSWVersion |
| Trace32 | cmm/init_debug.cmm; cmm/test_debug.cmm; cmm/gateway_debug.cmm; cmm/ota_debug.cmm; cmm/security_debug.cmm | 디버그 진입, breakpoint, watch 변수, interrupt/scheduler evidence capture script |
| CANoe | CANOE/6_ECUs_dbc_0206.dbc; CANOE/*.can; Configuration1.cfg | 6 ECU DBC, ECU simulator CAPL, Gateway/OTA CAPL test, panel bridge 및 binary report artifact |

# 빌드 및 실행 검증 결과

Debug_FLASH/Automotive_Cluster.elf(2,342,192 bytes)와 Debug_FLASH/Automotive_Cluster.map(594,806 bytes)이 확인되었다.

map 파일에서 vCANTask, OTA_HandleRequestMessage, COM_GatewayBuildStatusMessage, vComSchedulerTask 심볼이 확인되었다.

현재 셸 PATH에서 powerpc-eabivle-gcc 및 make는 확인되지 않아 Codex가 S32DS rebuild를 수행하지 않았다.

S32DS flash/run 10분 안정성, UART boot log, Trace32 screenshot, CANoe readable verdict export는 아직 확인 필요이다.

# 테스트 검증 결과

| 테스트 | 명령/도구 | 결과 | 관련 요구사항 |
| --- | --- | --- | --- |
| Host unit tests | python -m unittest discover -s Tests/host -p "test_*.py" -v | 9/9 pass | REQ-COM-005/007/015, REQ-ECU6-002/003/004/009/010/011/013/015, REQ-INT-007/008 |
| Gateway CAPL tests | CANOE/ECU6_Gateway_Test.can | 테스트 자산 및 binary report 존재, readable verdict 필요 | REQ-COM-003/011, REQ-ECU6-001/002/003/006/014, REQ-INT-009 |
| OTA/security CAPL tests | CANOE/ECU6_OTA_State_Test.can | 테스트 자산 및 binary report 존재, readable verdict 필요 | REQ-ECU6-005~013/015, REQ-INT-007/008 |

# 미확인/증거 부족 항목

REQ-COM-001, REQ-COM-003, REQ-COM-004, REQ-INT-001은 구현/자산 증거는 있으나 최종 런타임 증거가 부족하여 부분 충족으로 판정하였다.

ECU1~ECU5 개별 요구사항 56개와 REQ-INT-002~006은 현재 워크스페이스의 ECU6 중심 증거만으로 충족 판정할 수 없다.

CANoe .vtestreport 파일은 존재하지만 텍스트 검색으로 PASS/FAIL verdict를 확인할 수 없어 최종 판정 증거로 사용하지 않았다.

# 결론

Automotive_Cluster 워크스페이스는 ECU6 Gateway + OTA + Security 활성 범위의 대부분 요구사항을 코드/문서/host unit test 수준에서 충족한다. 다만 전체 96개 요구사항을 모두 충족한다고 증명하기에는 ECU1~ECU5 보드 구현 증거와 일부 통합 실행 증거가 부족하다. 따라서 본 보고서는 31개 충족, 4개 부분 충족, 61개 확인 필요로 판정한다.

# 부록: 파일 경로, 로그 요약, 테스트 명령, 주요 설정값

| 항목 | 값 |
| --- | --- |
| 요구사항 원문 | MPC5748G_Embedded_Training_Requirements_EN.docx |
| Host test log | Documentation\ecu6_gateway\evidence\host_unit_tests_2026-06-04.txt |
| Host test command | C:\Users\JunsangS\.cache\codex-runtimes\codex-primary-runtime\dependencies\python\python.exe -m unittest discover -s Tests/host -p \"test_*.py\" -v |
| CAN ID set | 0x101, 0x201, 0x301, 0x401, 0x501, 0x601, 0x650, 0x651 |
| 핵심 설정 | CAN_NODE_COUNT=6, CAN_TX_PERIOD_MS=100, LOG_BUFFER_SIZE=256, configTOTAL_HEAP_SIZE=32768 |