MPC5748G Automotive Cluster & Gateway Training Project

Detailed Requirement Specification

| Field | Description |
| --- | --- |
| Document type | Technical requirement specification |
| Target platform | 6 x MPC5748G boards, CANoe VN5650, Trace32, 3 x Arduino sensor emulator boards |
| Training goal | Enable mentees to understand, implement, debug, test, and integrate automotive embedded software on MPC5748G. |
| System concept | Six ECU nodes connected through CAN, monitored and tested by CANoe, debugged with Trace32, with Arduino-based sensor emulation. |
| Version | 1.0.0 |
| Date | 2026-05-18 |

# Document Control

| Version | Date | Author/Owner | Change Summary |
| --- | --- | --- | --- |
| v1.0.0 | 2026-05-19 | Mentor group | Initial version |

# Table of Contents

Contents

Document Control	2

Table of Contents	3

1. Project Overview and Training Scope	4

2. System Architecture	4

3. Requirement Conventions	5

4. Common Requirements for All Mentees	5

4.1 Common Software Structure	6

5. ECU1 - Body Control Node Requirements	6

Required deliverable folder	7

6. ECU2 - Powertrain Node Requirements	8

Required deliverable folder	9

7. ECU3 - Sensor Fusion Node Requirements	9

Required deliverable folder	11

8. ECU4 - Cluster Display Node Requirements	11

Required deliverable folder	12

9. ECU5 - Diagnostic and UDS Node Requirements	13

Required deliverable folder	14

10. ECU6 - Gateway, OTA, and Security Node Requirements	15

Required deliverable folder	17

11. Integration Scenarios	17

12. CAN Matrix	18

13. CANoe, Trace32, and Arduino Expectations	19

13.1 CANoe expectations	19

13.2 Trace32 expectations	19

13.3 Arduino expectations	19

14. Required Deliverables and Folder Structure	19

15. Evaluation Rubric and Review Checklist	20

15.1 Mentor review checklist	20

Appendix A - Suggested UDS DID and DTC Table	21

Appendix B - Recommended Final Demo Flow	21

# 1. Project Overview and Training Scope

This document defines the detailed technical requirements for a six-mentee automotive embedded training project using MPC5748G boards. Each mentee owns one ECU role. The six ECUs must communicate over CAN and integrate into a simulated vehicle cluster and gateway system.

The project is intentionally designed so that the ECU roles are different. This prevents direct copying while still forcing system-level integration. The expected implementation effort is approximately one focused week for coding and debugging, followed by integration, CANoe testing, Trace32 review, and final demonstration.

Mentees must understand hardware initialization, UART logging, GPIO, timers, CAN communication, error handling, and state-machine design.

Mentees must use CANoe VN5650 for CAN trace analysis, DBC-based signal decoding, panel creation, CAPL tests, and fault injection.

Mentees must use Trace32 to debug the target, load code, set breakpoints, inspect variables/registers, and create reusable CMM scripts.

Arduino boards are used as external sensor emulators through I2C/SPI and may also provide UART-based log viewing or input injection.

Advanced roles include UDS diagnostics, DTC handling, gateway filtering, OTA update simulation, and software-based security/HSM mock behavior.

| ECU | Owner | Role | Difficulty | Main Skills |
| --- | --- | --- | --- | --- |
| ECU1 | Mentee 1 | Body Control Node | 1.5/5 | GPIO, CAN Tx/Rx, door/seatbelt logic, turn/hazard signals, CANoe panel |
| ECU2 | Mentee 2 | Powertrain Node | 2.5/5 | Sensor input, RPM/speed processing, overheat logic, limp mode, fault handling |
| ECU3 | Mentee 3 | Sensor Fusion Node | 3/5 | I2C/SPI, filtering, range validation, timeout and stuck-sensor detection |
| ECU4 | Mentee 4 | Cluster Display Node | 3/5 | Multiple CAN Rx messages, warning lamps, state machine, fail-safe display |
| ECU5 | Mentee 5 | Diagnostic/UDS Node | 4/5 | UDS services, sessions, DID, DTC, negative responses, security access |
| ECU6 | Mentee 6 | Gateway + OTA + Security Node | 5/5 | Gateway monitoring/filtering, checksum/alive validation, OTA protocol, security mock |

# 2. System Architecture

The system consists of six MPC5748G boards connected to the same CAN network. CANoe/VN5650 acts as the simulation and test environment. Trace32 is used for target debug and automation. Three Arduino boards provide simulated sensor data and optional UART monitoring.

CANoe VN5650 <-> CAN Bus <-> ECU1 / ECU2 / ECU3 / ECU4 / ECU5 / ECU6

Trace32 <-> each MPC5748G board for debug, flashing, breakpoints, and CMM automation

Arduino 1 <-> ECU1/ECU4: body sensor emulator

Arduino 2 <-> ECU2/ECU6: powertrain sensor emulator and optional debug input

Arduino 3 <-> ECU3/ECU5: environment sensor emulator and optional diagnostic trigger

| Component | Role | Minimum Expectation |
| --- | --- | --- |
| MPC5748G board x6 | Main embedded ECU nodes | Boot, UART log, CAN Tx/Rx, application logic, Trace32 debug support |
| CANoe VN5650 | CAN simulation and testing | DBC decode, panels, CAPL test cases, trace logging, fault injection |
| Trace32 | Debug and automation tool | CMM scripts, reset/load/run, breakpoints, watch variables, memory/register inspection |
| Arduino x3 | Sensor emulator boards | I2C/SPI sensor data, UART log bridge, sensor fault injection |

# 3. Requirement Conventions

| Keyword | Meaning |
| --- | --- |
| MUST | Mandatory. The mentee cannot pass the project review without satisfying this requirement. |
| SHOULD | Strongly recommended. Missing this may reduce score but may not block integration. |
| MAY | Optional enhancement or bonus feature. |
| P0 | Critical for system integration and final demonstration. |
| P1 | Important for individual evaluation and maintainability. |
| P2 | Advanced or bonus requirement. |

Each requirement includes an ID, priority, requirement statement, acceptance criteria, and required evidence. Evidence can be source code, CANoe trace, CAPL result, UART log, Trace32 screenshot, CMM script, design document, or live demonstration.

# 4. Common Requirements for All Mentees

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-COM-001 | P0 | Each ECU MUST initialize system clock, GPIO, UART, CAN controller, timer/scheduler, and required interrupts. | The board boots successfully, prints a boot log, toggles a heartbeat LED, and runs for at least 10 minutes without abnormal reset. | Initialization source code, UART log, live demo. |
| REQ-COM-002 | P0 | Each ECU MUST provide UART logging with a consistent format: [time][ECUx][module][level] message. | Boot, init, CAN_TX, CAN_RX, application state, warning, and error events are visible. UART logging must not flood the system. | UART terminal screenshot or saved log file. |
| REQ-COM-003 | P0 | Each ECU MUST transmit at least one periodic CAN message and receive at least one CAN message from another ECU or CANoe. | CANoe trace shows the message, DBC decodes all mandatory signals, and receive processing is observable. | CANoe trace screenshot, DBC file, source code. |
| REQ-COM-004 | P0 | Each ECU MUST implement a 10 ms, 100 ms, and 1000 ms task or an equivalent scheduler. | Tasks execute periodically; no long blocking delay is used in the main control loop. | Scheduler code, Trace32 breakpoint evidence. |
| REQ-COM-005 | P0 | Each ECU MUST implement timeout monitoring for critical received CAN messages or external sensor inputs. | When input stops beyond the timeout threshold, the ECU sets an error flag, sends error status on CAN, and logs the event. | Fault injection test, CANoe trace. |
| REQ-COM-006 | P0 | Each ECU primary status message MUST include a 4-bit alive counter. | Alive counter increments from 0 to 15 and wraps to 0. Gateway/CANoe can detect if it stops changing. | DBC signal and CANoe trace/graph. |
| REQ-COM-007 | P0 | Each ECU primary status message MUST include a simple checksum. | Checksum is calculated consistently; CANoe or ECU6 can detect a deliberately corrupted checksum. | Checksum function, CAPL test. |
| REQ-COM-008 | P1 | Each ECU SHOULD separate driver, service, and application layers. | Main.c is not overloaded with all logic; modules are clearly separated. | Source tree review. |
| REQ-COM-009 | P1 | Each ECU SHOULD implement a state machine when application behavior has multiple modes. | States and transitions are clear, logged, and testable. | State machine design document and code. |
| REQ-COM-010 | P0 | Each mentee MUST create init_debug.cmm and test_debug.cmm for Trace32. | Mentor can connect, reset, load ELF, set breakpoints, and inspect variables using the scripts. | CMM files and Trace32 screenshot. |
| REQ-COM-011 | P0 | Each mentee MUST create at least three CAPL test cases for the assigned ECU. | At least one normal case, one invalid input case, and one timeout/fault case must produce PASS/FAIL result. | CAPL file and CANoe test result. |
| REQ-COM-012 | P0 | Each mentee MUST provide design and test documentation. | The documentation explains role, inputs, outputs, CAN messages, logic, error handling, and test evidence. | design.md and test_report.md. |
| REQ-COM-013 | P1 | Each ECU SHOULD support fault recovery when inputs or messages return to valid condition. | Error flags clear according to defined recovery conditions, and the ECU returns to normal operation. | Recovery test evidence. |
| REQ-COM-014 | P1 | Each ECU SHOULD use traceable names and comments linking important logic to requirement IDs. | Mentor can map code and tests back to requirements. | Code review. |
| REQ-COM-015 | P2 | Each ECU MAY include host-side unit tests for pure logic such as checksum, filters, warning rules, and state transitions. | Unit tests can be executed without hardware. | Unit test source and execution log. |

## 4.1 Common Software Structure

project_root/

drivers/

can_driver.c

uart_driver.c

gpio_driver.c

timer_driver.c

spi_driver.c or i2c_driver.c

services/

logger.c

checksum.c

timeout_monitor.c

uds.c / ota.c / security.c when applicable

app/

app_main.c

app_state_machine.c

app_signal_process.c

cmm/

init_debug.cmm

test_debug.cmm

# 5. ECU1 - Body Control Node Requirements

ECU1 simulates body-control functions including door status, seatbelt status, turn signal, hazard signal, and body warning output. It is the easiest ECU and focuses on GPIO, periodic CAN transmission, simple application logic, and CANoe panel interaction.

Input requirements

| Input | Source | Range / Encoding | Notes |
| --- | --- | --- | --- |
| Door_FL, Door_FR, Door_RL, Door_RR | Arduino or CANoe | 0 = closed, 1 = open | Invalid values must be rejected and logged. |
| Seatbelt | Arduino or CANoe | 0 = not fastened, 1 = fastened | Used for seatbelt warning logic. |
| TurnSignalMode | CANoe panel or Arduino switch | 0 = off, 1 = left, 2 = right, 3 = hazard | Controls LED blinking and CAN status. |
| VehicleSpeed | ECU2 or CANoe | 0..220 km/h | Used for door/seatbelt warning logic. |

Output requirements

| Output | Message / Interface | Cycle / Trigger | Content |
| --- | --- | --- | --- |
| 0x101 | BodyStatus | 100 ms | Door bits, Seatbelt, TurnSignalMode, HazardActive, BodyWarning, BodyInputError, AliveCounter, Checksum |
| GPIO LEDs | Body indicators | Event/periodic | Door warning, seatbelt warning, left/right blink, hazard, error indicator |
| UART | Debug log | Event/periodic | Boot, input update, CAN Tx/Rx, warning activation, errors |

Detailed requirements

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-ECU1-001 | P0 | ECU1 MUST read or receive four door states and validate each value. | CANoe/Arduino can set each door open/closed; invalid values set BodyInputError. | CANoe panel, UART log, test case. |
| REQ-ECU1-002 | P0 | ECU1 MUST read or receive seatbelt status and include it in BodyStatus. | Seatbelt state changes are reflected in CANoe within one BodyStatus cycle. | CANoe trace. |
| REQ-ECU1-003 | P0 | ECU1 MUST support turn signal modes: off, left, right, and hazard. | Left/right LEDs blink at approximately 500 ms; hazard blinks both sides. | Live demo and UART log. |
| REQ-ECU1-004 | P0 | ECU1 MUST send BodyStatus on CAN ID 0x101 every 100 ms. | CANoe measures stable 100 ms cycle and decodes all signals. | CANoe trace and DBC. |
| REQ-ECU1-005 | P0 | ECU1 MUST set BodyWarning if vehicle speed is greater than 5 km/h and any door is open. | Speed 40 km/h + Door_FL open results in BodyWarning = 1. Speed 0 may remain normal. | CAPL test TC_ECU1_DoorOpenDriving. |
| REQ-ECU1-006 | P0 | ECU1 MUST set BodyWarning if speed is greater than 10 km/h and seatbelt is not fastened. | Speed 20 km/h + Seatbelt = 0 results in BodyWarning = 1. | CAPL test TC_ECU1_SeatbeltWarning. |
| REQ-ECU1-007 | P1 | ECU1 SHOULD debounce physical/simulated switch inputs. | Short glitches do not cause unstable CAN signals. | Debounce test or code review. |
| REQ-ECU1-008 | P0 | ECU1 MUST detect input timeout if the selected input source is not updated for more than 500 ms. | Input timeout sets BodyInputError and logs the error. | Timeout test. |
| REQ-ECU1-009 | P1 | ECU1 SHOULD recover from input timeout when valid input resumes. | BodyInputError clears after three consecutive valid samples. | Recovery test. |
| REQ-ECU1-010 | P1 | ECU1 SHOULD provide a Body Control CANoe panel. | Panel can control door, seatbelt, turn mode and display warning/error flags. | Panel screenshot. |

Required CANoe panel elements

| CANoe panel element | Purpose |
| --- | --- |
| Door_FL checkbox | Control or display for this ECU |
| Door_FR checkbox | Control or display for this ECU |
| Door_RL checkbox | Control or display for this ECU |
| Door_RR checkbox | Control or display for this ECU |
| Seatbelt checkbox | Control or display for this ECU |
| TurnSignalMode dropdown | Control or display for this ECU |
| BodyWarning indicator | Control or display for this ECU |
| BodyInputError indicator | Control or display for this ECU |

Minimum CAPL test cases

| CAPL test case | Minimum expected result |
| --- | --- |
| TC_ECU1_001_DoorOpenWhileDriving | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU1_002_SeatbeltWarning | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU1_003_TurnSignalBlink | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU1_004_InputTimeout | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU1_005_ChecksumAliveCounter | PASS/FAIL result must be reported in CANoe test execution |

## Required deliverable folder

ecu1_body_node/

src/

inc/

cmm/

init_debug.cmm

test_debug.cmm

canoe/

panel file

CAPL test file

docs/

design.md

test_report.md

# 6. ECU2 - Powertrain Node Requirements

ECU2 simulates a powertrain node. It processes RPM, throttle, and engine temperature, calculates vehicle speed, detects overheating, and enters limp mode when a critical fault occurs.

Input requirements

| Input | Source | Range / Encoding | Notes |
| --- | --- | --- | --- |
| EngineRPM | Arduino or CANoe | 0..8000 rpm | Out-of-range RPM must be rejected. |
| ThrottlePercent | Arduino or CANoe | 0..100 % | Used for optional acceleration behavior. |
| EngineTemp | Arduino or CANoe | -40..150 degC | Warning > 105 degC, critical > 120 degC. |
| Reset/Fault commands | CANoe | Event-based | Used for fault injection and recovery. |

Output requirements

| Output | Message / Interface | Cycle / Trigger | Content |
| --- | --- | --- | --- |
| 0x201 | PowertrainStatus | 50 ms | VehicleSpeed, EngineRPM, EngineTemp, ThrottlePercent, EngineWarning, EngineCriticalFault, LimpModeActive, InputError, AliveCounter, Checksum |
| UART | Debug log | Event/periodic | RPM/speed/temp update, limp mode, overheat, input error |
| Optional GPIO LEDs | Fault indicators | Event | Engine warning and critical fault indicators |

Detailed requirements

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-ECU2-001 | P0 | ECU2 MUST receive or read EngineRPM, ThrottlePercent, and EngineTemp. | All values can be changed from CANoe or Arduino and observed in UART/CAN output. | CANoe panel and trace. |
| REQ-ECU2-002 | P0 | ECU2 MUST validate sensor ranges before using values. | RPM > 8000, throttle > 100, or temp outside -40..150 sets InputError and does not update calculations. | Invalid sensor CAPL test. |
| REQ-ECU2-003 | P0 | ECU2 MUST calculate VehicleSpeed from RPM or accepted input using a documented formula. | Example: RPM 2000 produces approximately 50 km/h if VehicleSpeed = RPM / 40. Speed is limited to 220 km/h. | Code and CANoe graph. |
| REQ-ECU2-004 | P1 | ECU2 SHOULD smooth speed changes instead of jumping instantly to target speed. | Speed gradually increases/decreases based on acceleration/deceleration step. | CANoe graph. |
| REQ-ECU2-005 | P0 | ECU2 MUST send PowertrainStatus on CAN ID 0x201 every 50 ms. | CANoe decodes all required signals with stable cycle time. | CANoe trace. |
| REQ-ECU2-006 | P0 | ECU2 MUST set EngineWarning when EngineTemp > 105 degC. | EngineTemp 110 degC results in EngineWarning = 1. | CAPL overheat test. |
| REQ-ECU2-007 | P0 | ECU2 MUST set EngineCriticalFault when EngineTemp > 120 degC. | EngineTemp 125 degC results in EngineCriticalFault = 1. | CAPL critical fault test. |
| REQ-ECU2-008 | P0 | ECU2 MUST enter limp mode when EngineCriticalFault is active. | VehicleSpeed is limited to <= 30 km/h while critical fault is active. | Limp mode test. |
| REQ-ECU2-009 | P0 | ECU2 MUST detect sensor timeout after 500 ms without valid input update. | InputError is set and VehicleSpeed ramps down to 0 or safe value. | Timeout test. |
| REQ-ECU2-010 | P1 | ECU2 SHOULD provide a recovery rule for overheat and sensor timeout. | Fault clears only after temperature/input returns valid for a defined number of cycles. | Recovery test. |

Required CANoe panel elements

| CANoe panel element | Purpose |
| --- | --- |
| RPM slider | Control or display for this ECU |
| Throttle slider | Control or display for this ECU |
| EngineTemp slider | Control or display for this ECU |
| VehicleSpeed display | Control or display for this ECU |
| EngineWarning indicator | Control or display for this ECU |
| CriticalFault indicator | Control or display for this ECU |
| LimpMode indicator | Control or display for this ECU |
| InjectInvalidRPM button | Control or display for this ECU |
| SensorTimeout button | Control or display for this ECU |

Minimum CAPL test cases

| CAPL test case | Minimum expected result |
| --- | --- |
| TC_ECU2_001_NormalSpeed | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU2_002_OverheatWarning | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU2_003_CriticalFault | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU2_004_LimpMode | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU2_005_InvalidSensor | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU2_006_SensorTimeout | PASS/FAIL result must be reported in CANoe test execution |

## Required deliverable folder

ecu2_powertrain_node/

src/

inc/

cmm/

init_debug.cmm

test_debug.cmm

canoe/

panel file

CAPL test file

docs/

design.md

test_report.md

# 7. ECU3 - Sensor Fusion Node Requirements

ECU3 collects and validates simulated environmental and chassis sensor data. It must demonstrate I2C/SPI communication, filtering, range checking, timeout monitoring, and stuck-sensor detection.

Input requirements

| Input | Source | Range / Encoding | Notes |
| --- | --- | --- | --- |
| AmbientTemp | Arduino or CANoe | -40..85 degC | Used for environment status. |
| LightLevel | Arduino or CANoe | 0..100 % | Night mode request when < 20%. |
| BatteryVoltage | Arduino or CANoe | 6..18 V | Low < 11 V, over-voltage > 15 V. |
| SteeringAngle | Arduino or CANoe | -540..+540 deg | Signed value must be handled correctly. |

Output requirements

| Output | Message / Interface | Cycle / Trigger | Content |
| --- | --- | --- | --- |
| 0x301 | SensorStatus | 100 ms | AmbientTemp, LightLevel, BatteryVoltage, SteeringAngle, BatteryLow, BatteryOverVoltage, NightModeRequest, SensorError, SensorStuck, AliveCounter, Checksum |
| UART | Debug log | Event/periodic | Raw/filtered sensor values, validation result, timeout/stuck detection |
| Optional LEDs | Sensor/fault indicators | Event | Low battery, night mode, sensor error |

Detailed requirements

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-ECU3-001 | P0 | ECU3 MUST receive or read ambient temperature, light level, battery voltage, and steering angle. | Each value can be controlled from Arduino/CANoe and is visible in SensorStatus. | CANoe trace. |
| REQ-ECU3-002 | P0 | ECU3 MUST validate range for all sensor inputs. | Out-of-range values set SensorError and are not used as valid output. | Invalid range CAPL test. |
| REQ-ECU3-003 | P0 | ECU3 MUST apply a moving average filter to at least one sensor. | Filtered value changes more smoothly than raw value; raw and filtered values are loggable. | UART log and code review. |
| REQ-ECU3-004 | P0 | ECU3 MUST send SensorStatus on CAN ID 0x301 every 100 ms. | CANoe decodes signed steering angle and all fault signals correctly. | DBC and CANoe trace. |
| REQ-ECU3-005 | P0 | ECU3 MUST set NightModeRequest when LightLevel < 20 %. | LightLevel 10 sets NightModeRequest = 1; LightLevel 50 clears it. | CAPL night mode test. |
| REQ-ECU3-006 | P0 | ECU3 MUST set BatteryLow when BatteryVoltage < 11 V. | Battery 10.5 V sets BatteryLow = 1. | Battery low test. |
| REQ-ECU3-007 | P0 | ECU3 MUST set BatteryOverVoltage when BatteryVoltage > 15 V. | Battery 16 V sets BatteryOverVoltage = 1. | Over-voltage test. |
| REQ-ECU3-008 | P0 | ECU3 MUST detect sensor timeout when no update is received for more than 500 ms. | Timeout sets SensorError and logs the source that timed out. | Timeout test. |
| REQ-ECU3-009 | P1 | ECU3 SHOULD detect a stuck sensor when a value remains unchanged for a configured number of samples while source is active. | FreezeSensor test sets SensorStuck = 1. | Stuck sensor test. |
| REQ-ECU3-010 | P1 | ECU3 SHOULD document I2C/SPI frame format used with Arduino. | Mentor can reproduce sensor emulator communication. | Interface document. |

Required CANoe panel elements

| CANoe panel element | Purpose |
| --- | --- |
| AmbientTemp slider | Control or display for this ECU |
| LightLevel slider | Control or display for this ECU |
| BatteryVoltage slider | Control or display for this ECU |
| SteeringAngle slider | Control or display for this ECU |
| FreezeSensor button | Control or display for this ECU |
| InvalidBattery button | Control or display for this ECU |
| SensorError indicator | Control or display for this ECU |
| SensorStuck indicator | Control or display for this ECU |
| NightModeRequest indicator | Control or display for this ECU |

Minimum CAPL test cases

| CAPL test case | Minimum expected result |
| --- | --- |
| TC_ECU3_001_NormalSensor | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU3_002_LowBattery | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU3_003_OverVoltage | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU3_004_NightMode | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU3_005_SensorStuck | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU3_006_SensorTimeout | PASS/FAIL result must be reported in CANoe test execution |

## Required deliverable folder

ecu3_sensor_fusion_node/

src/

inc/

cmm/

init_debug.cmm

test_debug.cmm

canoe/

panel file

CAPL test file

docs/

design.md

test_report.md

# 8. ECU4 - Cluster Display Node Requirements

ECU4 acts as the vehicle cluster. It receives status messages from other ECUs, displays speed/RPM/warnings, manages cluster states, and enters fail-safe mode when critical CAN messages are missing.

Input requirements

| Input | Source | Range / Encoding | Notes |
| --- | --- | --- | --- |
| BodyStatus 0x101 | ECU1 | Door, seatbelt, body warning | Timeout > 300 ms. |
| PowertrainStatus 0x201 | ECU2 | Speed, RPM, temp, engine faults | Timeout > 200 ms. |
| SensorStatus 0x301 | ECU3 | Battery, night mode, sensor fault | Timeout > 300 ms. |
| DiagnosticStatus 0x501 | ECU5 | Diagnostic active/session/DTC count | Used for DIAG_MODE. |
| GatewayStatus 0x601 | ECU6 | OTA/security/gateway warning | Used for OTA_MODE or warning display. |

Output requirements

| Output | Message / Interface | Cycle / Trigger | Content |
| --- | --- | --- | --- |
| 0x401 | ClusterStatus | 100 ms | ClusterState, DisplayedSpeed, DisplayedRPM, warning lamps, DiagModeLamp, OTAModeLamp, FailSafeLamp, AliveCounter, Checksum |
| UART | Cluster display log | 500 ms or 1 s | Readable display of speed, RPM, state, warnings |
| GPIO LEDs | Warning lamps | Event/periodic | Door, seatbelt, engine, battery, sensor, diag, OTA, fail-safe |

Detailed requirements

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-ECU4-001 | P0 | ECU4 MUST receive BodyStatus, PowertrainStatus, SensorStatus, DiagnosticStatus, and GatewayStatus. | Each message updates the related cluster display data. | CANoe trace and UART display. |
| REQ-ECU4-002 | P0 | ECU4 MUST implement a cluster state machine with INIT, NORMAL, WARNING, DIAG_MODE, OTA_MODE, and FAIL_SAFE states. | Mentor can trigger each state from CANoe or other ECUs; state is logged and sent in ClusterStatus. | State machine document and demo. |
| REQ-ECU4-003 | P0 | ECU4 MUST send ClusterStatus on CAN ID 0x401 every 100 ms. | CANoe decodes cluster state, displayed values, lamps, alive counter, and checksum. | CANoe trace. |
| REQ-ECU4-004 | P0 | ECU4 MUST display VehicleSpeed and EngineRPM from ECU2. | Speed/RPM in UART or LEDs matches PowertrainStatus within one update period. | CANoe/UART evidence. |
| REQ-ECU4-005 | P0 | ECU4 MUST turn on DoorWarningLamp when body warning indicates door open while driving. | Door open at speed results in DoorWarningLamp = 1. | Integration test. |
| REQ-ECU4-006 | P0 | ECU4 MUST turn on EngineWarningLamp for ECU2 EngineWarning or CriticalFault. | EngineTemp fault from ECU2 appears on cluster. | Engine warning test. |
| REQ-ECU4-007 | P0 | ECU4 MUST turn on BatteryWarningLamp for low battery or over-voltage from ECU3. | Battery 10.5 V or 16 V results in lamp ON. | Battery warning test. |
| REQ-ECU4-008 | P0 | ECU4 MUST enter FAIL_SAFE if PowertrainStatus timeout exceeds 200 ms. | When ECU2 stops sending, DisplayedSpeed becomes 0 and FailSafeLamp = 1. | Timeout test. |
| REQ-ECU4-009 | P0 | ECU4 MUST enter DIAG_MODE when DiagnosticStatus indicates active diagnostic operation. | UDS session/test from ECU5 triggers DiagModeLamp. | Diagnostic integration test. |
| REQ-ECU4-010 | P0 | ECU4 MUST enter OTA_MODE when GatewayStatus indicates active OTA operation. | OTA running on ECU6 triggers OTAModeLamp and OTA_MODE. | OTA integration test. |
| REQ-ECU4-011 | P1 | ECU4 SHOULD prioritize displays when multiple warnings are active. | Critical/fail-safe warnings have higher priority than normal warning display. | Priority logic document and test. |
| REQ-ECU4-012 | P1 | ECU4 SHOULD implement readable UART display updated every 500 ms or 1 s. | UART output is human-readable and not spammed. | UART screenshot. |

Required CANoe panel elements

| CANoe panel element | Purpose |
| --- | --- |
| Speed display | Control or display for this ECU |
| RPM display | Control or display for this ECU |
| ClusterState display | Control or display for this ECU |
| DoorWarning lamp | Control or display for this ECU |
| SeatbeltWarning lamp | Control or display for this ECU |
| EngineWarning lamp | Control or display for this ECU |
| BatteryWarning lamp | Control or display for this ECU |
| SensorWarning lamp | Control or display for this ECU |
| DiagMode lamp | Control or display for this ECU |
| OTAMode lamp | Control or display for this ECU |
| FailSafe lamp | Control or display for this ECU |

Minimum CAPL test cases

| CAPL test case | Minimum expected result |
| --- | --- |
| TC_ECU4_001_NormalDisplay | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU4_002_DoorWarning | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU4_003_EngineWarning | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU4_004_BatteryWarning | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU4_005_DiagnosticMode | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU4_006_OTAMode | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU4_007_PowertrainTimeout | PASS/FAIL result must be reported in CANoe test execution |

## Required deliverable folder

ecu4_cluster_node/

src/

inc/

cmm/

init_debug.cmm

test_debug.cmm

canoe/

panel file

CAPL test file

docs/

design.md

test_report.md

# 9. ECU5 - Diagnostic and UDS Node Requirements

ECU5 implements a basic diagnostic ECU. It must parse UDS requests from CANoe, handle sessions, DIDs, DTCs, clear diagnostics, ECU reset, security access, positive responses, and negative responses.

Input requirements

| Input | Source | Range / Encoding | Notes |
| --- | --- | --- | --- |
| UDS Physical Request | CANoe ID 0x7E0 | Event-based | Primary diagnostic request ID. |
| UDS Functional Request | CANoe ID 0x7DF | Event-based | Functional diagnostic request if supported. |
| Fault signals | ECU1/ECU2/ECU3/ECU6 | Event/periodic | Used to create DTC status. |
| CANoe diagnostic panel | Tester input | Event-based | Manual diagnostic operations. |

Output requirements

| Output | Message / Interface | Cycle / Trigger | Content |
| --- | --- | --- | --- |
| UDS Response | CAN ID 0x7E8 | Event | Positive and negative diagnostic responses |
| 0x501 | DiagnosticStatus | 100 ms | CurrentSession, SecurityUnlocked, ActiveDTCCount, LastServiceID, LastNRC, DiagActive, AliveCounter, Checksum |
| UART | Diagnostic log | Event | Received service, DID, DTC status, response code |

Detailed requirements

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-ECU5-001 | P0 | ECU5 MUST receive UDS physical requests on CAN ID 0x7E0 and respond on 0x7E8. | CANoe sends request and receives valid response frame. | CANoe diagnostic trace. |
| REQ-ECU5-002 | P0 | ECU5 MUST support Diagnostic Session Control service 0x10 for Default Session 0x01 and Extended Session 0x03. | 0x10 0x01 returns 0x50 0x01; 0x10 0x03 returns 0x50 0x03; unsupported sub-function returns NRC 0x12. | CAPL session tests. |
| REQ-ECU5-003 | P0 | ECU5 MUST support ECU Reset service 0x11 with hard reset sub-function 0x01 or a documented reset simulation. | Positive response is sent before reset/re-initialization; session returns to default after reset. | ECU reset test. |
| REQ-ECU5-004 | P0 | ECU5 MUST support Read Data By Identifier service 0x22. | CANoe can read supported DIDs and receives NRC 0x31 for unsupported DID. | ReadDID tests. |
| REQ-ECU5-005 | P0 | ECU5 MUST provide DID 0xF187 Software Version and DID 0xF188 Hardware Version. | DID values are returned consistently and documented. | CANoe test and DID table. |
| REQ-ECU5-006 | P1 | ECU5 SHOULD provide DID 0xF190 VIN. | If ISO-TP is not implemented, VIN may be shortened or documented as a simplified demo DID. | CANoe test. |
| REQ-ECU5-007 | P0 | ECU5 MUST support Write Data By Identifier service 0x2E for at least one demo configuration DID. | Write is rejected in Default Session and accepted in Extended Session. | WriteDID CAPL test. |
| REQ-ECU5-008 | P0 | ECU5 MUST support Read DTC Information service 0x19 0x02. | Active DTCs can be read after a fault is injected. | DTC test. |
| REQ-ECU5-009 | P0 | ECU5 MUST support Clear Diagnostic Information service 0x14 for all DTCs. | After clearing, ReadDTC reports no active DTC or zero count. | ClearDTC test. |
| REQ-ECU5-010 | P0 | ECU5 MUST implement negative responses for unsupported service, unsupported sub-function, request out of range, and conditions not correct. | CANoe tests receive correct 0x7F responses. | Negative response tests. |
| REQ-ECU5-011 | P1 | ECU5 SHOULD implement Security Access service 0x27 using a simple seed/key algorithm. | Correct key unlocks security; wrong key returns negative response. | Security access test. |
| REQ-ECU5-012 | P0 | ECU5 MUST send DiagnosticStatus on CAN ID 0x501 every 100 ms. | Cluster can detect diagnostic mode and CANoe displays current session/DTC count. | CANoe trace. |
| REQ-ECU5-013 | P1 | ECU5 SHOULD store DTCs based on fault status from ECU2, ECU3, or ECU6. | Engine overheat creates PowertrainOverheat DTC; sensor invalid creates SensorInvalid DTC. | Integration test. |
| REQ-ECU5-014 | P1 | ECU5 SHOULD include uds_debug.cmm for Trace32. | Script sets breakpoints in parser and watches session/security/DTC variables. | CMM file and screenshot. |

Required CANoe panel elements

| CANoe panel element | Purpose |
| --- | --- |
| Default Session button | Control or display for this ECU |
| Extended Session button | Control or display for this ECU |
| ECU Reset button | Control or display for this ECU |
| Read SW Version button | Control or display for this ECU |
| Read HW Version button | Control or display for this ECU |
| Read VIN button | Control or display for this ECU |
| Read DTC button | Control or display for this ECU |
| Clear DTC button | Control or display for this ECU |
| Request Seed button | Control or display for this ECU |
| Send Key button | Control or display for this ECU |
| Last Response display | Control or display for this ECU |
| Current Session display | Control or display for this ECU |
| SecurityUnlocked indicator | Control or display for this ECU |

Minimum CAPL test cases

| CAPL test case | Minimum expected result |
| --- | --- |
| TC_ECU5_001_DefaultSession | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_002_ExtendedSession | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_003_ReadDID | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_004_UnsupportedDID | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_005_WriteDID_DefaultReject | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_006_WriteDID_ExtendedOK | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_007_ReadDTC | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_008_ClearDTC | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_009_SecurityAccessOK | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU5_010_SecurityAccessFail | PASS/FAIL result must be reported in CANoe test execution |

## Required deliverable folder

ecu5_diagnostic_node/

src/

inc/

cmm/

init_debug.cmm

test_debug.cmm

canoe/

panel file

CAPL test file

docs/

design.md

test_report.md

# 10. ECU6 - Gateway, OTA, and Security Node Requirements

ECU6 is the most advanced ECU. It acts as a gateway monitor/filter, validates alive counters and checksums, supports OTA update simulation, and implements a software-based security/HSM mock for OTA authorization and message authentication.

Input requirements

| Input | Source | Range / Encoding | Notes |
| --- | --- | --- | --- |
| CAN messages from all ECUs | CAN bus | Periodic/event | Gateway must monitor ID, alive counter, checksum, and routing rules. |
| OTA Request 0x650 | CANoe | Event-based | Start, data, end, activate, abort commands. |
| Security request | CANoe | Event-based | Seed/key unlock sequence before OTA. |
| Invalid message injection | CANoe | Event-based | Used to test filtering and security error handling. |

Output requirements

| Output | Message / Interface | Cycle / Trigger | Content |
| --- | --- | --- | --- |
| 0x601 | GatewayStatus | 100 ms | GatewayState, OTAState, SecurityUnlocked, SecurityLocked, AliveError, ChecksumError, BlockedMsgCnt, SecurityErrorCounter, ActiveSWVersion, AliveCounter, Checksum |
| 0x651 | OTA_Response | Event | OTA ACK/NACK/verify result/activate response |
| UART | Gateway/OTA/security log | Event/periodic | Routing decision, OTA state, CRC, security result, blocked messages |

Detailed requirements

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-ECU6-001 | P0 | ECU6 MUST monitor or route allowed CAN IDs 0x101, 0x201, 0x301, 0x401, and 0x501. | Valid messages are accepted and reflected in gateway status/diagnostics. | CANoe trace and source code. |
| REQ-ECU6-002 | P0 | ECU6 MUST block or count unknown CAN IDs. | Injecting an unknown CAN ID increments GatewayBlockedMsgCnt. | CAPL invalid ID test. |
| REQ-ECU6-003 | P0 | ECU6 MUST validate checksum for monitored primary messages. | Wrong checksum sets GatewayChecksumError and increments blocked/error counter. | Checksum error test. |
| REQ-ECU6-004 | P0 | ECU6 MUST monitor alive counters for selected ECUs. | If alive counter does not change for N cycles, GatewayAliveError is set. | Alive counter freeze test. |
| REQ-ECU6-005 | P0 | ECU6 MUST implement OTA states: IDLE, START, RECEIVING, VERIFYING, READY_TO_ACTIVATE, SUCCESS, FAILED. | CANoe can trigger and observe each state through GatewayStatus. | OTA state-machine test. |
| REQ-ECU6-006 | P0 | ECU6 MUST support OTA_REQUEST CAN ID 0x650 and OTA_RESPONSE CAN ID 0x651. | CANoe sends OTA command and receives ACK/NACK response. | CANoe OTA trace. |
| REQ-ECU6-007 | P0 | ECU6 MUST support OTA commands START, DATA, END, ACTIVATE, and ABORT. | Valid command sequence completes successfully; invalid sequence returns negative response. | OTA protocol tests. |
| REQ-ECU6-008 | P0 | ECU6 MUST store OTA data in a bounded buffer of at least 256 bytes. | Normal 100-byte update succeeds; overflow attempt is rejected safely. | OTA overflow test. |
| REQ-ECU6-009 | P0 | ECU6 MUST verify OTA data using CRC/checksum before activation. | Correct CRC results in VERIFY_OK; wrong CRC results in OTA_FAILED. | OTA CRC tests. |
| REQ-ECU6-010 | P0 | ECU6 MUST require security unlock before accepting OTA_START. | OTA_START before unlock is rejected; OTA_START after correct key is accepted. | Security/OTA CAPL test. |
| REQ-ECU6-011 | P0 | ECU6 MUST implement seed/key security access for OTA. | Correct key unlocks; wrong key increments SecurityErrorCounter. | Security access test. |
| REQ-ECU6-012 | P1 | ECU6 SHOULD implement OTA_DATA authentication byte. | Data chunk with wrong AuthByte is rejected. | Auth test. |
| REQ-ECU6-013 | P1 | ECU6 SHOULD lock OTA after three failed security attempts. | After three wrong keys/auth failures, SecurityLocked = 1 and OTA is rejected until reset or timeout. | Security lock test. |
| REQ-ECU6-014 | P0 | ECU6 MUST send GatewayStatus on CAN ID 0x601 every 100 ms. | CANoe and ECU4 can decode gateway, OTA, and security status. | CANoe trace. |
| REQ-ECU6-015 | P1 | ECU6 SHOULD update ActiveSWVersion after successful OTA activation. | ActiveSWVersion changes only after verified OTA and activate command. | OTA success test. |
| REQ-ECU6-016 | P1 | ECU6 SHOULD provide gateway_debug.cmm, ota_debug.cmm, and security_debug.cmm. | Scripts set breakpoints and watch key variables for gateway, OTA, and security logic. | CMM files and screenshot. |

Required CANoe panel elements

| CANoe panel element | Purpose |
| --- | --- |
| Request Seed button | Control or display for this ECU |
| Send Correct Key button | Control or display for this ECU |
| Send Wrong Key button | Control or display for this ECU |
| OTA Start button | Control or display for this ECU |
| Send OTA Chunk button | Control or display for this ECU |
| OTA End Correct CRC button | Control or display for this ECU |
| OTA End Wrong CRC button | Control or display for this ECU |
| OTA Activate button | Control or display for this ECU |
| OTA Abort button | Control or display for this ECU |
| OTA State indicator | Control or display for this ECU |
| SecurityUnlocked indicator | Control or display for this ECU |
| SecurityLocked indicator | Control or display for this ECU |
| BlockedMessageCount display | Control or display for this ECU |
| GatewayChecksumError indicator | Control or display for this ECU |

Minimum CAPL test cases

| CAPL test case | Minimum expected result |
| --- | --- |
| TC_ECU6_001_GatewayAllowValidMessage | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_002_GatewayBlockInvalidID | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_003_GatewayBlockWrongChecksum | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_004_SecurityUnlockOK | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_005_SecurityUnlockFail | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_006_OTARejectWithoutSecurity | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_007_OTASuccess | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_008_OTACRCFail | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_009_OTAOverflow | PASS/FAIL result must be reported in CANoe test execution |
| TC_ECU6_010_SecurityLockAfter3Failures | PASS/FAIL result must be reported in CANoe test execution |

## Required deliverable folder

ecu6_gateway_ota_security_node/

src/

inc/

cmm/

init_debug.cmm

test_debug.cmm

canoe/

panel file

CAPL test file

docs/

design.md

test_report.md

# 11. Integration Scenarios

| Requirement ID | Priority | Requirement Statement | Acceptance Criteria | Evidence / Deliverable |
| --- | --- | --- | --- | --- |
| REQ-INT-001 | P0 | All six boards MUST run together on the same CAN network without CAN ID conflict. | CANoe sees messages 0x101, 0x201, 0x301, 0x401, 0x501, and 0x601 concurrently. | Full-system CANoe trace. |
| REQ-INT-002 | P0 | The system MUST support a Normal Driving scenario. | Speed 40 km/h, RPM 1800, temp 85 degC, doors closed, seatbelt fastened, battery 12.5 V results in Cluster NORMAL and no active warnings. | Integration test report. |
| REQ-INT-003 | P0 | The system MUST support Door Open While Driving scenario. | Speed 40 km/h + Door_FL open results in ECU1 BodyWarning and ECU4 DoorWarningLamp. | CANoe trace and panel screenshot. |
| REQ-INT-004 | P0 | The system MUST support Engine Overheat scenario. | EngineTemp 125 degC results in ECU2 CriticalFault, limp mode, ECU4 EngineWarningLamp, and ECU5 DTC storage. | CAPL integration test. |
| REQ-INT-005 | P0 | The system MUST support Sensor Fault scenario. | Battery 16 V or sensor timeout results in ECU3 SensorError/BatteryOverVoltage and ECU4 warning lamp. | Fault injection evidence. |
| REQ-INT-006 | P0 | The system MUST support Diagnostic scenario. | CANoe starts Extended Session, reads DID, reads/clears DTC, and ECU4 displays diagnostic mode. | UDS trace and CAPL result. |
| REQ-INT-007 | P0 | The system MUST support OTA Success scenario. | CANoe unlocks security, sends OTA data, verifies CRC, activates update, and ECU6 updates ActiveSWVersion. | OTA trace and GatewayStatus. |
| REQ-INT-008 | P0 | The system MUST support OTA Fail scenario. | Wrong CRC results in OTA_FAILED and ActiveSWVersion remains unchanged. | OTA negative test. |
| REQ-INT-009 | P0 | The system MUST support Gateway Security scenario. | Invalid ID or wrong checksum is blocked/counted by ECU6 and shown in CANoe panel. | Gateway CAPL test. |

# 12. CAN Matrix

| CAN ID | Sender | Receiver | Cycle | Message | Main Signals / Purpose |
| --- | --- | --- | --- | --- | --- |
| 0x101 | ECU1 | All/CANoe | 100 ms | BodyStatus | Door_FL/FR/RL/RR, Seatbelt, TurnSignalMode, HazardActive, BodyWarning, InputError, AliveCounter, Checksum |
| 0x201 | ECU2 | All/CANoe | 50 ms | PowertrainStatus | VehicleSpeed, EngineRPM, EngineTemp, ThrottlePercent, EngineWarning, CriticalFault, LimpMode, InputError, AliveCounter, Checksum |
| 0x301 | ECU3 | All/CANoe | 100 ms | SensorStatus | AmbientTemp, LightLevel, BatteryVoltage, SteeringAngle, BatteryLow, OverVoltage, NightModeRequest, SensorError, SensorStuck, AliveCounter, Checksum |
| 0x401 | ECU4 | All/CANoe | 100 ms | ClusterStatus | ClusterState, DisplayedSpeed, DisplayedRPM, warning lamps, DiagMode, OTAMode, FailSafe, AliveCounter, Checksum |
| 0x501 | ECU5 | All/CANoe | 100 ms | DiagnosticStatus | CurrentSession, SecurityUnlocked, ActiveDTCCount, LastServiceID, LastNRC, DiagActive, AliveCounter, Checksum |
| 0x601 | ECU6 | All/CANoe | 100 ms | GatewayStatus | GatewayState, OTAState, SecurityUnlocked, SecurityLocked, AliveError, ChecksumError, BlockedMsgCnt, SecurityErrorCounter, ActiveSWVersion, AliveCounter, Checksum |
| 0x650 | CANoe | ECU6 | Event | OTA_Request | Command, sequence, data bytes, CRC/AuthByte as defined by ECU6 |
| 0x651 | ECU6 | CANoe | Event | OTA_Response | ACK/NACK, verify OK/fail, activation result |
| 0x7DF | CANoe | ECU5/ECU6 | Event | UDS Functional Request | Functional diagnostic request |
| 0x7E0 | CANoe | ECU5 | Event | UDS Physical Request | Physical diagnostic request |
| 0x7E8 | ECU5 | CANoe | Event | UDS Response | Positive/negative diagnostic response |

# 13. CANoe, Trace32, and Arduino Expectations

## 13.1 CANoe expectations

CANoe configuration must include a DBC file with all project messages and signals.

Each ECU must have at least one related panel section or panel page.

Each ECU must have CAPL tests with clear PASS/FAIL reporting.

CANoe must be able to inject normal values, boundary values, invalid values, timeouts, and gateway/security attacks.

CANoe trace logs must be saved as evidence for final review.

## 13.2 Trace32 expectations

init_debug.cmm must connect to target, reset target, load ELF, set breakpoint at main, and run to main.

test_debug.cmm must set breakpoints for CAN Tx and CAN Rx and watch at least three ECU-specific variables.

Advanced ECUs must provide extra scripts such as uds_debug.cmm, ota_debug.cmm, gateway_debug.cmm, or security_debug.cmm.

CMM scripts should be repeatable by mentor without many manual steps.

## 13.3 Arduino expectations

Arduino firmware must document the I2C/SPI frame format if used.

Arduino must be able to produce normal sensor values and at least one fault condition.

Arduino UART output should help verify sensor values and communication status.

If Arduino is not available during a test, CANoe simulation may be used as fallback, but the interface design must still be documented.

# 14. Required Deliverables and Folder Structure

MPC5748G_Automotive_Cluster_Gateway_Project/

common/

can/

uart/

timer/

checksum/

logger/

ecu1_body_node/

ecu2_powertrain_node/

ecu3_sensor_fusion_node/

ecu4_cluster_node/

ecu5_diagnostic_node/

ecu6_gateway_ota_security_node/

arduino/

body_sensor_emulator/

powertrain_sensor_emulator/

environment_sensor_emulator/

canoe/

dbc/

panels/

capl_tests/

logs/

trace32/

common_scripts/

docs/

architecture.md

can_matrix.md

integration_test_report.md

| Deliverable | Mandatory Content |
| --- | --- |
| Source code | Drivers, services, application logic, ECU-specific modules, build configuration. |
| Header files | Public interfaces, signal structures, configuration constants, requirement-related enums. |
| CAN description | CAN IDs, cycle time, signals, scaling, alive counter, checksum, sender/receiver. |
| State machine document | States, transitions, events, error handling, recovery conditions. |
| CANoe evidence | Panel screenshot, trace screenshot, CAPL test result. |
| Trace32 evidence | CMM scripts, breakpoint/watch screenshot, optional memory/register dump. |
| Test report | Test objective, setup, steps, expected result, actual result, PASS/FAIL, evidence link. |
| Demo evidence | Live demo or video covering individual ECU and integration scenario. |

# 15. Evaluation Rubric and Review Checklist

| Category | Points | Review Focus |
| --- | --- | --- |
| Board bring-up, UART, GPIO, timer | 10 | Boot stability, heartbeat LED, UART logs, scheduler. |
| CAN communication and DBC correctness | 20 | Tx/Rx, cycle time, DBC decode, alive counter, checksum. |
| Sensor/CANoe/Arduino integration | 15 | Input source, range handling, fault injection, interface documentation. |
| Application logic and state machine | 20 | Correct behavior, state transitions, warning/fault logic, safe behavior. |
| Error handling and recovery | 10 | Timeout, invalid input, checksum/alive error, recovery behavior. |
| Trace32 usage and CMM scripts | 10 | Reusable CMM, breakpoints, watched variables, debug evidence. |
| CANoe panel and CAPL tests | 10 | Panel usability, automated tests, PASS/FAIL evidence. |
| Documentation and demo quality | 5 | Readable docs, traceability, final demo clarity. |

## 15.1 Mentor review checklist

[ ] Board boots successfully

[ ] UART log is readable

[ ] Heartbeat LED works

[ ] CAN Tx works

[ ] CAN Rx works

[ ] DBC decode is correct

[ ] Timeout handling exists

[ ] Invalid input handling exists

[ ] Application logic/state machine is clear

[ ] Trace32 CMM scripts work

[ ] CANoe panel exists

[ ] CAPL tests run with PASS/FAIL result

[ ] Documentation is complete

[ ] ECU integrates with at least two other boards

[ ] Full-system demo scenario passes

# Appendix A - Suggested UDS DID and DTC Table

| Identifier | Meaning | Owner | Notes |
| --- | --- | --- | --- |
| DID 0xF187 | Software Version | ECU5 / ECU6 | Returned by ReadDataByIdentifier. |
| DID 0xF188 | Hardware Version | ECU5 | Board or project hardware version. |
| DID 0xF190 | VIN or demo VIN | ECU5 | Can be shortened if ISO-TP is not implemented. |
| DID 0x1234 | Current Fault Status | ECU5 | Summarizes active system faults. |
| DID 0x1235 | Last Reset Reason | ECU5 | Useful for reset test. |
| DID 0x1236 | Demo Configuration | ECU5 | Writable only in Extended Session. |
| DTC 0x100001 | Body communication timeout | ECU5 | Set based on ECU1/body timeout status. |
| DTC 0x200001 | Powertrain overheat | ECU5 | Set when ECU2 critical overheat is active. |
| DTC 0x300001 | Sensor invalid | ECU5 | Set when ECU3 SensorError is active. |
| DTC 0x600001 | Gateway security error | ECU5 | Set when ECU6 reports security violation. |

# Appendix B - Recommended Final Demo Flow

Power on all six boards and verify UART boot logs.

Open CANoe configuration and confirm all periodic messages are visible and decoded.

Run Normal Driving scenario and verify Cluster NORMAL state.

Inject Door Open While Driving and verify BodyWarning and Cluster lamp.

Inject Engine Overheat and verify limp mode, Cluster warning, and DTC storage.

Run UDS ReadDID, ReadDTC, ClearDTC from CANoe.

Run OTA success sequence with security unlock and correct CRC.

Run OTA failure sequence with wrong CRC.

Inject invalid CAN ID/checksum and verify Gateway blocks or counts it.

Use Trace32 CMM on at least one ECU to show breakpoint/watch automation.